| Operations/Work/Job Activity covered by this assessment: Final-Inspection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Personal Protective Equipment (PPE) : Safety glasses must be worn at times.; Long and loose hair must be contained.. Appropriate footwear with substantial uppers must be worn.; Close fitting/protective clothing to cover arms and legs must be worn.. Respiratory protection devices required.; Oil free leather gloves and spats must be worn.. Hearing protection must be used when using this machine.. |
| Loading & Unloading Check: - Ensure no slip/trip is present in work pass and walk ways. - Ensure that work area is clean and clear of grease, oil and any flammable materials. - Refer Work Instructions PSP/WI/14 while handling. - Refer VT Procedure PSP/PM/22 While Visual Inspection. |  |  |  |  |  |
| Operations Procedure: # Scope: To check whether the Component coated and finished as per customer requirements. # Take out print of FIR (Final-Inspection Report) format No. QF/QA/11 from software. # FIR indicates the Customer Name, Part Name, Qty etc… # Carry out Visual Inspection Document No. QF/QMS/00/26 to check scratch, dents, pinholes etc.., Checking & Recording the dimension, coating length and total length of the job. # Dimension to check using Calibrated Instruments like, Vernier Calipers and Micrometers. Always check for error and mention instrument no. on FIR. # Compare the component with Pre-inspection Report/ Drawing given by the Customer. # Fill the before coating dimension, after coating dimension, calculate the thickness observed and check for the value required by the customer, also check the surface roughness and enter the value. # Record components details like, punch no and mention any remark/observation in the remark cells. # If Deviation is found contact your QA/Production Engineer/ Works Manager, and get clearance from the Customer by sending Deviation Report. # After getting the Deviation approval from the Customer the Production Engineer/ Works Manager will approve. # After approval, the FIR is marked completed and saved in Software. # After Final Inspection Send FIR & TC to Customer. |  |  |  |  |  |
| Assessor : |  | Signature: |  | Date: |  |
| Approved: |  | Signature: |  | Date: |  |