| Operations/Work/Job Activity covered by this assessment: Pre-Inspection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Personal Protective Equipment (PPE) : Safety glasses must be worn at times.; Long and loose hair must be contained.. Appropriate footwear with substantial uppers must be worn.; Close fitting/protective clothing to cover arms and legs must be worn.. Respiratory protection devices required.; Oil free leather gloves and spats must be worn.. Hearing protection must be used when using this machine.. |
| Loading & Unloading Check: - Ensure no slip/trip is present in work pass and walk ways. - Ensure that work area is clean and clear of grease, oil and any flammable materials. - Refer Work Instructions PSP/WI/14 while handling. - Refer VT Procedure PSP/PM/22 While Visual Inspection. |  |  |  |  |  |
| Operations Procedure: # Scope: To check whether the Component received from the customer is conformal to its requirements and defect free for further processing. # Check Schedule in Dashboard /Email & collect PIR from Admin. # Check PIR Comment Section , If job is our Manufacturing 100% inspection to be done. # Refer Code - Pre inspection code. In case of absence of quote one can proceed without code and give PR to ISR for making quotation. # Compare the component with Customer inspection Report & Drawings. In case any deviation must be escalated as per escalation matrix . # Also refer Inspection code document no QF-QA-26 for method of checking and cross-checking PIR format no. # Strictly Follow the checklist mentioned on the PIR to avoid missing any fundamental points and avoid delays and errors in job execution. # Carry out Visual Inspection as per VT Procedure Document No. PSP/PM/22 # In case of any dents/Scratches found during inspection, QC Personnel must get clearance from Customer via email before proceeding further. # Dimension to be check using Calibrated Instruments. Always check for error and mention instrument no. on PIR. # Check if job is below 50 HRC, if above. Ensure that checking hardness should not damage coating area of job. For ex – like barrel of Rolls. # Compare the component with Customer inspection Report & Drawings. In case drawing not available make hand sketch and indicate dimensions. # In case of coating removals of job, a follow-up inspection is required post coating removal and same to be mentioned on PIR # Any extra work scope required to be mentioned on PIR and highlighted for easy identification of internal Sales representative. # Get clearance from Production Engineer/Works Manager for De-coating/Grinding/ Machining /Welding of the component according to the dimension size and customer specification. # Record components details like, punch no, As Received dimension, Customer Dimension, Deviation etc. # After getting the approval from the QA / customer the QC Inspector will approve the PIR. In case of regular and responsive customer QC to coordinate directly with customers # After approval, the PIR is marked completed and hand over to internal sales Representative. # In case of any unavailability of information, nonconformance, queries one can contact QA / ISR or raise query in daily meeting to resolve in timely manner. |  |  |  |  |  |
| Assessor : | Prassana Wadkar | Signature: | PSW | Date: | 01.04.2026 |
| Approved: | Haarsh Doshi | Signature: | HDD | Date: | 01.04.2026 |