| Operations/Work/Job Activity covered by this assessment: Handling Customer Components |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Personal Protective Equipment (PPE) : Safety glasses must be worn at times.; Long and loose hair must be contained.. Appropriate footwear with substantial uppers must be worn.; Close fitting/protective clothing to cover arms and legs must be worn.. Respiratory protection devices required.; Oil free leather gloves and spats must be worn.. Hearing protection must be used when using this machine.. |
| Loading & Unloading Check: - Ensure no slip/trip is present in work pass and walk ways. - Ensure that work area is clean and clear of grease, oil and any flammable materials - Ensure that customer components (e.g. rolls, rotors, screws, etc.) is properly tighten by rope during Loading & Unloading. |  |  |  |  |  |
| Operations Procedure: - When you receive a component from the customer, see whether the Wooden Box , Protective Packing of the job is intact, if it is damaged or broken; inform the Supervisor/Production Engineer/Works Manager. - Ensure the handling equipment such as sling, hooks and hoist are in proper condition. - Lift with the help of EOT/ Hydra/ Forklift and place it very carefully at the designated area. - While lifting, positioning, placing, ensure the component is properly balanced and slings and wire ropes are properly placed. - Ensure the component does not collide with any other component/object to prevent damage or any injury, while positioning/placing. - Avoid keeping the component without packing on to the floor or kept on Wooden Plank. - Make sure that the component is handled carefully from the time it enters the works till the time of dispatch. - Take proper care of the component while packing, lifting, and loading while dispatching. - Keep the slings at the designated areas. |  |  |  |  |  |
| Assessor : |  | Signature: |  | Date: |  |
| Approved: |  | Signature: |  | Date: |  |