| Operations/Work/Job Activity covered by this assessment: Repairing/Recoating of Customer Components |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Required Personal Protective Equipment (PPE) : Safety glasses must be worn at times.; Long and loose hair must be contained.. Appropriate footwear with substantial uppers must be worn.; Close fitting/protective clothing to cover arms and legs must be worn.. Respiratory protection devices required.; Oil free leather gloves and spats must be worn.. Hearing protection must be used when using this machine.. |
| Loading & Unloading Check: - Ensure no slip/trip is present in work pass and walkways. - Ensure that work area is clean and clear of grease, oil and any flammable materials. - Refer Work Instructions PSP/WI/14 while handling. |  |  |  |  |  |
| Operations Procedure: - Refer Pre-Inspection Work Instructions PSP/QMS/02/03 to pre-inspect the Component. - Visually check the components for any damages, like dents, scratches, pin holes etc.. inform customer accordingly. - If the components have come for recoating, remove the old coating by means of Al2O3 80 mesh Grit to achieve a clean parent metal substrate. - Inspect the component and note dimension in the pre-inspection report. - Look for defects like pin holes, dents etc. if any, report to the Quality inspector. - Polish the component to observe any deformity on the surface. - Check the actual dimension after polishing and record it. - Inform Customer and take approval for repairing of the components. - Repair the defects with the help of spot welding/buffing/cut on a lathe, grinding etc. to make the component fit for coating. - Check dimension and record. # Refer Final Blasting Work Instructions PSP/QMS/02/04 Rev 01 for preparing surface. - Refer Spraying Process instruction (HVOF. Plasma, TWAS). - Refer PSP/QMS/02/12 and PSP/QMS/02/13 for finishing of the component. - Refer Final Inspection PSP/QMS/02/10. |  |  |  |  |  |
| Assessor : |  | Signature: |  | Date: |  |
| Appr |  | Signature: |  | Date: |  |