# Operations/Work/Job Activity covered by this assessment: Packing Procedure

Required Personal Protective Equipment (PPE) : Safety glasses must be worn at times.; Long and loose hair must be contained.. Appropriate footwear with substantial uppers must be worn.; Close fitting/protective clothing to cover arms and legs must be worn.. Respiratory protection devices required.; Oil free leather gloves and spats must be worn.. Hearing protection must be used when using this machine..

Loading & Unloading Check: - Ensure no slip/trip is present in work pass and walkways. - Ensure that work area is clean and clear of grease, oil and any flammable materials. - Refer Work Instructions PSP/WI/14 while handling.

Operations Procedure: 1. Handling During Loading/Unloading - The Packing Department must ensure all customer components are handled with utmost care during loading and unloading. - Components must be placed securely in wooden or cardboard boxes to prevent any physical damage. 2. Coordination with Quality Department - Before packing any outgoing components, obtain the Packing Slip/List from the Quality Department. - This ensures traceability and proper documentation of all dispatched items. 3. Storage on Shop Floor - When storing customer components on the shop floor, racks, or stands: - Use rubber sheets or wooden planks to avoid direct contact with hard surfaces. - This prevents scratches, dents, or any damage to the components. 4. Packing Method – Standard Components - Wrap all customer components using the following four-layer protection: - Rust Inhibitor Paper / Stretch Film (first layer) - Bubble Wrap or Foam Sheet (second layer) - Cotton Navar Strip (third layer) - Outer Film (fourth layer) - During the rainy season, add an additional plastic sheet to protect against moisture. 5. Use of Customer-Supplied Boxes - If using customer-provided boxes: - Inspect thoroughly for any damage or wear before use. - Do not dispatch components in compromised packaging. 6. Pre-Strapping Quality Check - Before final strapping of boxes: - Inform the Quality Department. - Allow quality personnel to verify the arrangement of components inside the box. - Ensure photographs are taken for records. 7. Final Packing Checklist - After packing: - Record weight and dimensions on the Packing Slip. - Inform the Office Personnel. - Collect and affix the Packing Label/List on the wooden or cardboard box. 8. Labelling Unboxed Components - For components sent without wooden/cardboard boxes: - Wrap in film and cotton navar as per standard. - Affix a label indicating Job Number or Customer ID Number. 9. Packing Heavy Components - For heavy components: - Apply three layers of standard wrapping (as above). - Secure with 1-inch Jute Rope. - Apply Plastic Sheet as the final, fourth layer. Technical Specifications Material; Specification. Foam Sheet; 1 mm thickness. Cotton Navar; 2” width × 1.85 mm thickness. Stretch Film; 3 microns. Plastic Sheet; 20 microns. Rust Preventive Oil; SOVCL 412.