Operations/Work/Job Activity covered by this assessment: Cutting Machine

Required Personal Protective Equipment (PPE) : Safety glasses must be worn at times.; Long and loose hair must be contained.. Appropriate footwear with substantial uppers must be worn.; Close fitting/protective clothing to cover arms and legs must be worn.. Respiratory protection devices required.; Oil free leather gloves and spats must be worn.. Hearing protection must be used when using this machine..

C F E B D A G H C F E B D A G H - Switch on the main supply (A) - Switch the power in the machine (B) - Mount the circular specimen in the vice and align with the circular blade, close the safety door. - Switch on the emergency button. (C) - Press the green button and the cutting knob (D). It glows green. - Set the RPM knob to desired speed of cutting wheel (E) - With the help of lever (H), slowly bring the rotating wheel to the specimen. - Press the coolant button (G) - Start cutting the specimen by slowly lowering the lever. - After cutting the specimen fully, shift the right-hand side lever(H) to the upward position and then switch off the motor. - Switch `OFF’ the power button on the machine (B) - Switch `OFF’ main switch (A)